| Nitte Meenakshi Institute of Technology (AN AUTONOMOUS INSTITUTION AFFILIATED TO VISVESVARAYA TECHNOLOGICAL UNIVERSITY, BELGAUM) PB No. 6429, Yelahanka, Bangalore 560-064, Karnataka Telephone: 080- 22167800, 22167860 Fax: 080 - 22167805. Department of Information Science and Engineering |
| --- |
| SMID-SEM EXAMINATION-II Semester/ Section: 3 -A/B/C Date: 04/02/2023 Subject and Code: Digital Design (21IS36) Duration: 1.0 Hr. Faculty: Dr. Swarnalatha K S, Mr. Pramod Jain, Mrs. Vani E S Max Marks: 30 Note: Answer any two full questions. |  |  |  |  |
| Q. No |  | Questions | Marks | CO/BL |
| 1 | a | Explain the working of 2-bit magnitude comparator | 4 | 2/2 |
|  | b | Derive the Characteristic equations for SR, JK, D and T flip flops. | 6 | 3/3 |
|  | c | Assume, initial status of the D flip-flops from leftmost to rightmost is Q3Q2Q1Q0=0000. Here, Q3 & Q0 are MSB & LSB output of D FFs respectively. If Binary information “1011” is used i. To insert Serially and obtain it Parallel across Q3Q2Q1Q0, design the shift register to perform the above operation and find number of clock pulses required to store and retrieve the binary information. ii. Repeat the above to insert binary information Parallel and retrieve it serially across Q3Q2Q1Q0 | 5 | 4/3 |
| 2 | a | How asynchronous inputs are different from synchronous inputs | 4 | 4/2 |
|  | b | Design a positive edge triggered D flip flop Using S R flip flop. | 6 | 3/3 |
|  | c | What are Asynchronous Counters. Discuss the working of Mod-8 Asynchronous Up counter with a neat diagram | 5 | 4/2 |
| 3 | a | Define race around condition. With a neat diagram explain master slave JK flip flop | 4 | 3/2 |
|  | b | Design a MOD-6 synchronous counter using T-flip flop for the following Sequence: 0, 2, 3, 6, 5, 1, 0, 2…. | 6 | 4/3 |
|  | c | Design and implement 4:16 decoder using 2:4 decoders. List the drawbacks of encoder and how to overcome it.. | 5 | 2/3 |